- 通用诊断需求

  - 帧格式 （待澄清）

对于CANFD节点，只支持CANFD帧诊断请求，忽略CAN帧的诊断请求，且诊断响应也是CANFD格式。

1.2 CAN Identifier 标识符

标准标识符（11位）

  - DLC长度策略

支持CAN FD类型的诊断报文，在APP里，Tx_DLC和Rx_DLC均为8，即支持8个字节长度的诊断保文

1.4网络层支持最大字节数

一次可发送或接收最多4095个数据字节。

1.4诊断请求_物理寻址地址

对于RL雷达来说：CANID定义为0x785

对于RR雷达来说：CANID定义为0x786

1.4诊断请求_功能寻址地址

RL/RR 其CANID定义均为0x7DF。

1.5 诊断响应地址

对于RL雷达来说：CANID定义为0x78D

对于RR雷达来说：CANID定义为0x78E

1.6填充位

对应诊断请求报文，其填充位是0x55，对应诊断响应报文，其填充位是0xAA.

1.7流控帧Flow Control (FC)

流控帧支持三种状态：

- FC.CTS:continue to send

- FC.WAIT:the request to continue to wait

- FC.OVFLW:buffer overflow

1.8 BlockSize数据块大小

BS值为0.

1.9 STmin

APP: 5ms

BOOT: 0 ms

1.10 STmin保留位

当STmin保留位被设为80-F0（Reserved）和FA-FF(Reserved),雷达ECU应该按照127ms来响应，当STmin> N_CS,用N_CS代替STmin。

1.11 网络层时间参数

| Network Layer Timing Parameters 网络层定时器参数 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Symbol 符号 | Timeout 超时时间 | Performance Requirement 性能要求 | Unit 单位 |
| ECU 电控单元 | N_As/N_Ar | 250 | - | ms |
|  | N_Bs | 250 |  | ms |
|  | N_Br | - | <100 | ms |
|  | N_Cs | - | <100 | ms |
|  | N_Cr | 250 | - | ms |

N_Br/Cr(待确认)

1.12 会话层时间参数

| Application Layer Timing Parameters 应用层会话管理定时器参数 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Symbol 符号 | Min 最小值 | Max/Timeout 最大值/超时时间 | Unit 单位 |
| ECU 电控单元 | P2Server | 0 | 50 | ms |
|  | P2*Server | 0 | 5000 | ms |
|  | S3Server | N/A | 5000 | ms |

1.13通信设计

采用半双工的通信模式，同一时刻两节点之间的点对点通信只能为单向。

1.14 特殊要求

1.14.1 NRC78的次数

NRC78最大数量为3.

2.诊断服务(肯定响应抑制位各服务支持情况，待确认)

2.1 ECU诊断会话转换图

2.2 0x10

2.2.1 10 01默认会话

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 |  | Sub-functions 子功能(HEX) |  | Session/会话 |  |  |  |  |  | Security Access 安全访问 |  |  |  |  |
|  |  |  |  |  | Default 默认 |  | Extended 扩展 |  |  |  | Level1 层级1 |  |  | Level2 层级2 |  |
|  |  |  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 |  | Func 功能 寻址 |  |  |  |  |  |  |
| 10 | DiagnosticSessionControl 诊断模式控制 |  | 01 DefaultSession/默认会话模式 |  | Y | Y | Y |  | Y |  | N |  |  | N |  |
| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) |  | Diagnostic Services Name 诊断服务名称 |  | Sub-functions 子功能(HEX) |  |  |  | Session/会话 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Default 默认 |  |  |  | Programming 编程 |  |  | Level3 层级3 |  |
|  |  |  |  |  |  |  |  | Phys 物理 寻址 |  | Func 功能 寻址 |  | Phys 物理 寻址 | Func 功能 寻址 |  |  |  |
| 10 |  | DiagnosticSessionControl 诊断模式控制 |  | 01 DefaultSession/默认会话模式 |  |  |  | Y |  | Y |  | Y | Y |  | N |  |

2.2.1.1上电之后，ECU的会话模式

当ECU上电之后，没有启动其他的诊断会话，应一直处于默认会话模式。

2.2.2 10 02编程会话

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 10 | DiagnosticSessionControl 诊断模式控制 | 02 ProgrammingSession/编程会话模式 | Y | N | Y | N | N | N |

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 10 | DiagnosticSessionControl 诊断模式控制 | 02 ProgrammingSession/编程会话模式 | N | N | Y | N | N |

2.2.3 10 03 扩展会话

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 10 | DiagnosticSessionControl 诊断模式控制 | 03 ExtendedDiagnosticSession/扩展诊断会话模式 | Y | Y | Y | Y | N | N |

2.3.4 NRC：

12/13/22（10 02独有）/7E

NRC22:车速大于10km/h不允许正确执行，返回NRC22否定码

2.3.5 NRC优先级（待澄清）

2.3 0x11

2.3.1 11 01硬件复位

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 11 | EcuReset 电控单元复位 | 01 hardReset/硬复位 | Y | Y | Y | Y | N | N |

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 11 | EcuReset 电控单元复位 | 01 hardReset/硬复位 | Y | Y | Y | Y | N |

2.3.2响应要求

如果肯定响应抑制位为0，肯定响应消息应在ECU执行复位之前发送。

2.3.3复位之后会话要求

ECU成功复位之后应激活默认会话模式。

2.3.4对存储数据的要求

对于应存入非易失性内的数据，ECU应确保在执行复位前完成保存操作 。 (待澄清有哪些数据)

2.3.5复位之后恢复通信时间（待澄清）

复位需要等待2sECU能够恢复通信.

2.3.6NRC：

12/13/22（10 02独有）

NRC22:车速大于10km/h不允许正确执行，返回NRC22否定码

2.3.7NRC优先级（待澄清）

2.4 0x14

2.4.1诊断属性要求

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 14 | ClearDiagnosticInformation 清除诊断信息 |  | Y | Y | Y | Y | N | N |

2.4.2GroupOfDTC:

0xFF FF FF ，代表 all group

2.4.3 NRC：

7F/13/31

2.4.4NRC优先级

（待澄清）

2.5 0x19

2.5.1 19 01

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 19 | ReadDTCInformation 读取DTC信息 | 01 reportNumberOfDTCByStatusMask/通过状态掩码报告DTC数量 | Y | Y | Y | Y | N | N |

2.5.2 19 02

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 19 | ReadDTCInformation 读取DTC信息 | 02 reportDTCByStatusMask/通过状态掩码报告DTC | Y | Y | Y | Y | N | N |

2.5.3 19 04

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 19 | ReadDTCInformation 读取DTC信息 | 04 reportDTCSnapshotRecordByDTCNumber/通过DTC报告DTC快照记录 | Y | Y | Y | Y | N | N |

2.5.4 19 06

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 19 | ReadDTCInformation 读取DTC信息 | 06 reportDTCExtendedDataRecordByDTCNumber/通过DTC报告DTC扩展数据记录 | Y | Y | Y | Y | N | N |

2.5.5 19 0A

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 19 | ReadDTCInformation 读取DTC信息 | 0A reportSupportedDTC/报告支持的DTC | Y | Y | Y | Y | N | N |

2.5.6支持DTC status

| Bit Field 位场 | Bit Description 描述 | Bit State Definition 位状态定义 | Supported by ECU1 ECU支持1 | Remarks 备注 |
| --- | --- | --- | --- | --- |
| 0 | Test failed 测试失效 | 0=False 1=True | Y |  |
| 1 | Test failed this monitoring cycle 本检测周期测试失效 | 0=False 1=True | Y |  |
| 2 | Pending DTC 未确认DTC | 0=False 1=True | Y |  |
| 3 | Confirmed DTC 确认DTC | 0=False 1=True | Y |  |
| 4 | Test not completed since last clear 上次清零后测试未完成 | 0=False 1=True | Y |  |
| 5 | Test failed since last clear 上次清零后测试失效 | 0=False 1=True | Y |  |
| 6 | Test not completed this monitoring cycle 本检测周期测试未完成 | 0=False 1=True | Y |  |
| 7 | Warning indicator requested 警告指示位请求 | 0=False 1=True | N |  |
| 要求：供应商须提供所支持DTCStatus bit由0-1及由1-0转化的详细策略。 |  |  |  |  |

Pending 位几周期置0（待澄清）

2.5.7支持的快照数据

| Snapshot Record Num 快照记录号 | Data Identifier 数据标识符 | Snapshot Record Description 快照记录描述 | Size (Bytes) 字节大小 | Range, Min (Phy.) 物理值最小值 | Range, Max (Phy.) 物理值最大值 | Unit 单位 | Conversion 换算 | Data Type 数据类型 | The filling value of the signal source fault | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 信号源故障时的填充值 |  |
| 01 | F010 | Year(high byte) | 01 | —— | —— | —— | 例如2019年，前两个字节为0x2019 | BCD | 1. 如果实时时间信号无法获取，使用最近一次值。 2.当ECU下电时，应将本操作循环最近一次实时时间值保存至非易失性内存。 3.如果ECU在任何操作循环都没有收到过实时时间信号，使用全0x99进行填充。 4.如果总线上的时间信号不符合时间编码规则，要求继续使用该值作为快照。 | DTC快照应分别记录故障第一次发生时刻和最近一次发生时刻的相关数据，分别存储在不同的快照记录码（DTCSnapshotRecordNumber）中，具体要求如下： （1）DTCSnapshotRecordNumber = 0x01，0x01组快照数据要求记录故障第一次发生时刻的相关数据，如果后续操作循环该故障一直存在（confirmed DTC（bit3=1）），其快照内容应保持不变。 （2）DTCSnapshotRecordNumber = 0x05，0x05组快照数据要求记录故障最近一次发生时刻的相关数据，如果当前或后续操作循环该故障再次发生（testFailed（bit0由0置1）），0x05组快照应当更新。 |
|  |  | Year(low byte) | 01 | —— | —— | —— |  | BCD |  |  |
|  |  | Month | 01 | —— | —— | —— | 例如12月，此字节为0x12 | BCD |  |  |
|  |  | Day | 01 | —— | —— | —— | 例如25号，此字节为0x25 | BCD |  |  |
|  |  | Hour | 01 | —— | —— | —— | 例如下午2点，此字节为0x14 | BCD |  |  |
|  |  | Minute | 01 | —— | —— | —— | 例如30分，此字节为0x30 | BCD |  |  |
|  |  | Second | 01 | —— | —— | —— | 例如30秒，此字节为0x30 | BCD |  |  |
|  | F011 | ECU电压 | 01 | 0 | 25.5 | V | E=N * 0.1 | USN |  |  |
|  | F013 | 车速 | 02 | 0 | 1023.984375 | km/h | E = N * 0.015625 | USN | 当车速信号丢失或者无效时使用0xFFFF填充 |  |
|  | F014 | 里程 | 04 | 0 | 67108863 | km | E = N * 0.015625 | USN | 1. 如果车辆里程信号无效或无法获取，使用最近一次有效值。 2. 当ECU下电时，应将本操作循环最近一次有效车辆里程值保存至非易失性内存。 3. 如果ECU在任何操作循环都没有收到过有效的车辆量程信号，使用全0xFF进行填充。 |  |
|  | 2601 | ECU Temperature | 01 | -50 | 120 | ℃ | Y=X-50 | USN |  |  |
| 05 | F010 | Year(high byte) | 01 | —— | —— | —— | 例如2019年，前两个字节为0x2019 | BCD | 1. 如果实时时间信号无法获取，使用最近一次值。 2.当ECU下电时，应将本操作循环最近一次实时时间值保存至非易失性内存。 3.如果ECU在任何操作循环都没有收到过实时时间信号，使用全0x99进行填充。 4.如果总线上的时间信号不符合时间编码规则，要求继续使用该值作为快照。 |  |
|  |  | Year(low byte) | 01 | —— | —— | —— |  | BCD |  |  |
|  |  | Month | 01 | —— | —— | —— | 例如12月，此字节为0x12 | BCD |  |  |
|  |  | Day | 01 | —— | —— | —— | 例如25号，此字节为0x25 | BCD |  |  |
|  |  | Hour | 01 | —— | —— | —— | 例如下午2点，此字节为0x14 | BCD |  |  |
|  |  | Minute | 01 | —— | —— | —— | 例如30分，此字节为0x30 | BCD |  |  |
|  |  | Second | 01 | —— | —— | —— | 例如30秒，此字节为0x30 | BCD |  |  |
|  | F011 | ECU电压 | 01 | 0 | 25.5 | V | E=N * 0.1 | USN |  |  |
|  | F013 | 车速 | 02 | 0 | 1023.984375 | km/h | E = N * 0.015625 | USN | 当车速信号丢失或者无效时使用0xFFFF填充 |  |
|  | F014 | 里程 | 04 | 0 | 67108863 | km | E = N * 0.015625 | USN | 1. 如果车辆里程信号无效或无法获取，使用最近一次有效值。 2. 当ECU下电时，应将本操作循环最近一次有效车辆里程值保存至非易失性内存。 3. 如果ECU在任何操作循环都没有收到过有效的车辆量程信号，使用全0xFF进行填充。 |  |
|  | 2601 | ECU Temperature | 01 | -50 | 120 | ℃ | Y=X-50 | USN |  |  |

2.5.7.1里程信息

2.5.7.2时间信息

2.5.8支持的扩展数据

| Extended Data Record Num 扩展数据记录号 | Extended Data Record Description 扩展数据记录描述 | Size (Bytes) | Range, Min (Phy.) 物理值最小值 | Range, Max (Phy.) 物理值最大值 |
| --- | --- | --- | --- | --- |
| 01 | Fault occurrence counter | 01 | 0 | 255 |
| 02 | Fault pending counter | 01 | 0 | 255 |
| 03 | DTC Aging counter | 01 | 0 | 40 |
| 04 | DTC Aged counter | 01 | 0 | 255 |

01：Fault occurrence counter

02：Fault pending counter

03：DTC Aging counter

04：DTC Aged counter

2.5.9支持的DTC

2.5.9.1RR_DTCList(network)

1.CAN 总线关闭（Bus Off）-C07300

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | U007300 | C07300 | CAN Bus Off error | CAN 总线关闭（Bus Off） | 通讯内核激活后1000ms开始 1000ms after ECU entering the Communication Kernels active state | 持续主动检测 | 连续3次发生总线关闭 Bus off has happened for consecutive 3 times | 记录DTC（报BUSOFF故障码不应再报节点丢失、报文丢失、数据无效性故障）） | 连续成功发送5帧报文 Consecutive 5 messages have been sent successfully |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

2.电源电压低 - 900016

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | B100016 | 900016 | Battery voltage low | 电源电压低 Battery voltage is too low | 1.通讯内核激活后开始监测 2.ECU休眠立即停止监测 | 持续主动检测 | 电压持续1秒小于9V Battery voltage has been consecutively under 9V for 1s | 不允许再进行其他故障（含电气和网络类）监测 Fault detection (including electrical fault and network fault) is prohibited | 电压持续1秒大于等于9.5V Battery voltage has been consecutively over 9.5V for 1s |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

3.电源电压高 - 900017

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | B100017 | 900017 | Battery voltage high | 电源电压高 Battery voltage is too high | ECU上电 KL30 node：Battery Connect KL15 node：IGN ON | 持续主动检测 | 电压持续1秒大于16V Battery voltage has been consecutively over 16V for 1s | 不允许再进行其他故障（含电气和网络类）监测 Fault detection (including electrical fault and network fault) is prohibited | 电压持续1秒小于等于15.5V Battery voltage has been consecutively under 15.5V for 1s |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

5.车速-timeout: D40287

| 序号 ind+D1+B6:J11 | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | U140287 | D40287 |  | 来自ABS的故障指示、EBD故障指示、EPS OFF指示、车身稳定系统状态、备用车速信息报文$17D丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个该报文周期未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

6.D40283

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | U140283 | D40283 |  | 来自ABS的故障指示、EBD故障指示、EPS OFF指示、车身稳定系统状态、备用车速信息报文$17D信号故障（running counter故障或checksum故障或invalid故障） | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个报文周期接收到报文running counter故障或checksum故障或invalid故障（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for 10 consecutive message periods(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的running counter或checksum或invalid错误 ECU has detected message without running counter/checksum /invalid fault for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

7.D40087

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | U140087 | D40087 |  | 来自ABS的工作状态、车身稳定系统变速箱挡位请求、牵引力控制系统、车身稳定系统、坡道保持系统工作状态报文$1E9丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

8.D40F81

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | U140F81 | D40F81 |  | 来自ESC的车辆横摆角速度信息报文$1E9无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x1E9 VehDynYawRateV=0x1:Invalid（检测到报文丢失不应报此故障） ECU has detected message 0x1E9 with VehDynYawRateV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文 0x1E9 VSELatAccV ！=0x1:Invalid ECU has detected message 0x1E9 with VSELatAccV=!0x1:Invalid for 5 consecutive message periods | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

9.E00487

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | U200487 | E00487 |  | 来自BCM的门锁、门碰状态信息报文$12A丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个该报文周期未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

10.E00087

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | U200087 | E00087 |  | 来自IBC的点火钥匙位置、手刹开关、变速箱倒挡开关状态信息报文$1F1丢失 | 开始监测：唤醒后1000ms开始 停止监测：ECU休眠 | 持续主动检测 | 连续10个该报文周期未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

11.E00083

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | U200083 | E00083 |  | 来自IBC的点火钥匙位置、手刹开关、变速箱倒挡开关状态信息报文$1F1信号故障（running counter故障或checksum故障或invalid故障） | 开始监测：唤醒后1000ms开始 停止监测：ECU休眠 | 持续主动检测 | 连续10个报文周期接收到报文running counter故障或checksum故障或invalid故障（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for 10 consecutive message periods(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的running counter或checksum或invalid错误 ECU has detected message without running counter/checksum /invalid fault for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

12.D91287

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | U191287 | D91287 |  | 来自HCU车速、整车状态、PHEV工作模式报文$201丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个该报文周期未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

13.E00387

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | U200387 | E00387 |  | 来自BCM的照明灯、指示灯状态信息报文$140丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个该报文周期未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

14.D60087

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | U160087 | D60087 |  | 来自EPS的故障码指示及工作状态信息、工作电流、方向盘角度报文$1E5丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

15.E00887

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | U200887 | E00887 |  | 来自IBC的电动窗控制状态、变道闪光设置状态、熄火自动解锁方式设置状态等信息报文$43D丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个该报文周期未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

16.D40387

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | U140387 | D40387 |  | 来自ABS的左、右驱动轮对地速度信息报文丢失$348 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

17.D40381

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17 | U140381 | D40381 |  | 来自ABS的左驱动轮对地速度信息报文$348失效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x348 LtFntWhlGrndVlctyV =0x1:invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x348 with LtFntWhlGrndVlctyV =0x1:invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x348 LtFntWhlGrndVlctyV !=0x1:invalid(无效) ECU has detected message 0x348 with LtFntWhlGrndVlctyV !=0x1:invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

18.D40381

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | U140781 | D40781 |  | 来自ABS的右驱动轮对地速度信息报文$348失效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x348 RtFntWhlGrndVlctyV=0x1:invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x348 with RtFntWhlGrndVlctyV=0x1:invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x348 RtFntWhlGrndVlctyV !=0x1:invalid(无效) ECU has detected message 0x348 with RtFntWhlGrndVlctyV !=0x1:invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

19.D40787

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | U140787 | D40787 |  | 来自车身稳定系统纵向加速度、主缸制动压力报文$128丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

20.D41F81

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | U141F81 | D41F81 |  | 来自ESC的车身稳定系统纵向加速度信息报文$128无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x128 VSELongAccV=0x1:Invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x128 with VSELongAccV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到该报文 VSELongAccV ！=0x1:Invalid(无效) ECU has detected message 0x128 with VSELongAccV ！=0x1:Invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

21.D40487

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | U140487 | D40487 |  | 来自ABS的左、右从动轮对地速度信息报文丢失$34A | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

22.D40481

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | U140481 | D40481 |  | 来自ABS的左从动轮对地速度信息报文$34A失效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x34A LtRrWhlGrndVlctyV =0x1:invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x34A with LtRrWhlGrndVlctyV =0x1:invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x34A LtRrWhlGrndVlctyV !=0x1:invalid(无效) ECU has detected message 0x34A with LtRrWhlGrndVlctyV !=0x1:invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

23.D40881

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | U140881 | D40881 |  | 来自ABS的右从动轮对地速度信息报文$34A失效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x34A RtRrWhlGrndVlctyV =0x1:invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x34A with RtRrWhlGrndVlctyV !=0x1:invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

24.D40483

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | U140483 | D40483 |  | 来自ESC的从动轮信息报文$34A信号故障（running counter故障或checksum故障或invalid故障） | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到该报文running counter故障或checksum故障或invalid故障（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的running counter或checksum或invalid错误 ECU has detected message without running counter/checksum /invalid fault for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

25.D40B87

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | U140B87 | D40B87 |  | 来自ESP的左、右非驱动轮脉冲及方向信息报文$0C5丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

26.D40B81

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | U140B81 | D40B81 |  | 来自ESC的左非驱动轮脉冲信息报文$0C5无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms接收到报文0x0C5 LtRrWhlRotStaWhlDstncV=0x1:Invalid（无效)（检测到报文丢失不应报此故障） ECU has detected message 0x0C5 with LtRrWhlRotStaWhlDstncV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x0C5 LtRrWhlRotStaWhlDstncV!=0x1:Invalid（无效） ECU has detected message 0x0C5 with LtRrWhlRotStaWhlDstncV!=0x1:Invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

27.D41B81

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27 | U141B81 | D41B81 |  | 来自ESC的右非驱动轮脉冲信息报文$0C5无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms接收到报文0x0C5 RtRrWhlRotStaWhlDstncV=0x1:Invalid（无效）（检测到报文丢失不应报此故障） ECU has detected message 0x0C5 with RtRrWhlRotStaWhlDstncV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x0C5 RtRrWhlRotStaWhlDstncV!=0x1:Invalid（无效） ECU has detected message 0x0C5 with RtRrWhlRotStaWhlDstncV!=0x1:Invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

28.D40A87

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | U140A87 | D40A87 |  | 来自ESP的左、右驱动轮脉冲及方向信息报文$0C1丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms未检测到该报文 | 记录DTC | 连续5个该报文周期检测到该报文 | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

29.D40A81

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29 | U140A81 | D40A81 |  | 来自ESC的左驱动轮脉冲信息报文$0C1无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到接收到报文0x0C1 LtFntWhlRotStaWhlDstncV=0x1:Invalid（无效）（检测到报文丢失不应报此故障） ECU has detected message 0x0C1 with LtFntWhlRotStaWhlDstncV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x0C1 LtFntWhlRotStaWhlDstncV!=0x1:Invalid（无效） ECU has detected message 0x0C1 with LtFntWhlRotStaWhlDstncV!=0x1:Invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

30.D41A81

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | U141A81 | D41A81 |  | 来自ESC的右驱动轮脉冲信息报文$0C1无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms接收到报文0x0C1 RrFntWhlRotStaWhlDstncV=0x1:Invalid（无效）（检测到报文丢失不应报此故障） ECU has detected message 0x0C1 with RrFntWhlRotStaWhlDstncV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x0C1 RrFntWhlRotStaWhlDstncV!=0x1:Invalid（无效） ECU has detected message 0x0C1 with RrFntWhlRotStaWhlDstncV!=0x1:Invalid for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

31.D40A83

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | U140A83 | D40A83 |  | 来自ESP的左、右驱动轮脉冲及方向信息报文$0C1的checksum或rolling counter错误 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续250ms检测到该报文checksum错误或rolling counter（检测到报文丢失不应报此故障） ECU has detected message with checksum fault or rolling counter fault for consecutive 250 ms detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的checksum错误或rolling counter错误 ECU has detected message without checksum and rolling counter fault for 5 consecutive message periode | / |  | 1、经过40次无故障点火循环后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 |

32.D91A87

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | U191A87 | D91A87 | Message of frame 341 such as vehicle SOC, Vehicle Actual Gear Status from VCU are lost | 来自HCU/VCU的整车SOC值、整车当前档位报文$341丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续10个报文周期未接收到该报文 ECU can’t detect this message for 10 consecutive message periods | 记录DTC | 连续5个报文周期接收到该报文 ECU has detected this message for 5 consecutive message periods | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

33.D94781

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | U194781 | D94781 |  | 来自HCU/VCU的报文$341整车当前档位状态有效性信号为无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续10个报文周期检测到接收到报文0x341 VehActGrStsV=0x1:Invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x341 with VehActGrStsV=0x1:Invalid for consecutive 10 consecutive message periods(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到该报文0x155 VehActGrStsV!=0x1:Invalid(无效) ECU has detected message 0x155 with VehActGrStsV!=0x1:Invalid for 5 consecutive message periods | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 3. ECU receives a command of clearing DTCs from the tester. |

34.D91E83

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | U191E83 | D91E83 |  | 来自HCU/VCU的整车当前档位状态有效性、整车当前档位状态报文$341checksum或rolling counter错误 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续10个周期检测到该报文checksum错误或rolling counter（检测到报文丢失不应报此故障） ECU has detected message with checksum fault or rolling counter fault for 10 consecutive times detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的checksum错误或rolling counter错误 ECU has detected message without checksum and rolling counter fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

35.C07307私CAN busoff

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | U007307 | C07307 | DTC_SFCAN_BUSOFF | 私CAN busoff | CR not in Drive Test Mode 雷达未处于驾驶测试模式 | 持续主动检测 | 连续3次发生私CAN关闭 Bus off has happened for consecutive 3 times | DTC's logged BSD/LCA off, RCTA off, RCTB off, DOW off, RCW off. | Immediately after there's no Bus off occur on Private CAN 没有私CAN 报文 bus off 出现 连续成功发送5帧报文 Consecutive 5 messages have been sent successfully |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

36.D18087

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | U118087 | D18087 | DTC_RADAR_TIMEOUT | RL雷达功能状态报文 timeout | 1) CR not in Drive Test Mode 雷达未处于驾驶测试模式 2) Ignition on for 3 second 点火3s 3) CR not in production mode 雷达不在生产模式 4) Voltage in normal range (9v-16v) 电压正常9v-16v 5) Can bus is not off CAN 通讯没有off | 持续主动检测 | Lost communication with the other side rear corner radar for 10 consecutive times 连续10个报文周期未接收到该报文 | DTC's logged BSD/LCA off, RCTA off, RCTB off, DOW off, RCW off. | Receive can message successfully for 5 times 成功接收到到报文5次 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

37.D40081

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37 | U140081 | D40081 | Vehicle Stability Enhancement Lateral Acceleration Invalidity | 来自ESC的车辆横向加速度信息报文$1E9无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到接收到报文0x1E9 VSELatAccV=0x1:Invalid（无效）（检测到报文丢失不应报此故障） ECU has detected message 0x1E9 with VSELatAccV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文 0x1E9 VSELatAccV ！=0x1:Invalid ECU has detected message 0x1E9 with VSELatAccV=!0x1:Invalid for 5 consecutive message periods | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

39.D40083

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | U140083 | D40083 | Checksum fault of frame $1E9 | 来自ABS的工作状态、车身稳定系统变速箱挡位请求、牵引力控制系统、车身稳定系统、坡道保持系统工作状态报文$1E9信号故障（running counter故障或checksum故障或invalid故障） | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到该报文checksum错误（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的running counter或checksum或invalid错误 ECU has detected message without running counter/checksum /invalid fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

40.D40281

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | U140281 | D40281 | Vehicle_Speed_Average_Driven_Invalid_$17D | 备用车速无效$17D | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续10个报文周期接收到报文0x17D VehSpdAvgDrvnV_ABS=0x1:Invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x17D with VehSpdAvgDrvnV_ABS=0x1:Invalid for 10 consecutive message periods | 记录DTC | 连续5个报文周期接收到该报文0x17D VehSpdAvgDrvnV_ABS !=0x1:Invalid(无效) ECU has detected message 0x17D with VehSpdAvgDrvnV_ABS !=0x1:Invalid for 5 consecutive message periods | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

41.D40383

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | U140383 | D40383 | Checksum fault of frame $348 | 来自ESC的驱动轮信息报文$348 checksum错误 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到该报文checksum错误（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的checksum错误 ECU has detected message without checksum fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

42.D40687

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42 | U140687 | D40687 | Message of frame $230 such as EPB working status from ESP are lost | 来自ESP的EPB工作状态报文$230丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms未接收到该报文 ECU can’t detect this message for consecutive 250 ms. | 记录DTC | 连续5个报文周期接收到该报文 ECU has detected this message for 5 consecutive message periods | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

43.D40B83

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43 | U140B83 | D40B83 | Checksum fault of frame $0C5 | 报文$0C5 checksum错误 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到该报文Checksum错误（检测到报文丢失不应报此故障） ECU has detected message with Checksum fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的checksum错误 ECU has detected message without checksum fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

44.D18183

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44 | U118183 | D18183 | DTC_RADAR_Func_ALV | RL雷达功能状态报文ALV | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个周期检测到该报文rolling counter错误（检测到报文丢失不应报此故障） ECU has detected message with rolling counter fault for 10 consecutive times(detecting the loss of the message, it should not be reported) | DTC's logged BSD/LCA off, RCTA off, RCTB off, DOW off, RCW off. | Check if the RRR_Func(RX)or RRL_Func(Rx) or RRL_Object(RX) is transmitting message(s) with correct Alivecounter. 确认RRR_Func(RX)or RRL_Func(Rx) Alivecounter 正确 连续5个报文周期未接收到该报文 rolling counter错误 ECU has detected message without rolling counter fault for 5 consecutive message periode |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

45.D18283

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45 | U118283 | D18283 | DTC_RADAR_Func_CRC | RL雷达功能状态报文CRC | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 | 连续10个周期检测到该报文checksum错误（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for 10 consecutive times detecting the loss of the message, it should not be reported) | DTC's logged BSD/LCA off, RCTA off, RCTB off, DOW off, RCW off. | Check if the RRR_Func(RX)or RRL_Func(Rx) or RRL_Object(RX) is transmitting message(s) with correct CRC. 确认RRR_Func(RX)or RRL_Func(Rx) CRC 正确 连续5个报文周期未接收到该报文 的checksum错误 ECU has detected message without checksum fault for 5 consecutive message periode |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

46.D60083

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | U160083 | D60083 | Checksum fault of frame $1E5 | 来自EPS的方向盘角度信号0x1E5报文 checksum错误 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到该报文checksum错误（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的checksum错误 ECU has detected message without checksum fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

47.E00483

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | U200483 | E00483 | rolling counter fault or Checksum fault of frame $12A | 来自IBC的门锁、门碰状态信息报文$12A信号故障（running counter故障或checksum故障或invalid故障） | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续10个报文周期接收到报文rolling counter错误（检测到报文丢失不应报此故障） ECU has detected message with rolling counter fault for 10 consecutive message periods(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的running counter或checksum或invalid错误 ECU has detected message without running counter/checksum /invalid fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 4. ECU receives a command of clearing DTCs from the tester. |

48.D60081

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48 | U160081 | D60081 | Steering Wheel Angle Invalidity | 来自EPS的方向盘转角信号有效性报文$1E5有效性无效 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到接收到报文0x1E5 StrWhAngV=0x1:Invalid(无效)（检测到报文丢失不应报此故障） ECU has detected message 0x1E5 with StrWhAngV=0x1:Invalid for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期接收到报文0x1E5 StrWhAngV !=0x1:Invalid(无效) ECU has detected message 0x1E5 with StrWhAngV !=0x1:Invalid for 5 consecutive message periods | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

49.D40783

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49 | U140783 | D40783 | Checksum fault of frame $128 | 来自ESC的制动缸压力信息报文$128 checksum错误或$128 rolling counter错误 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到该报文checksum错误（检测到报文丢失不应报此故障） ECU has detected message with checksum fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 的checksum错误或rolling counter错误 ECU has detected message without checksum and rolling counter fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

51.D91D87

| 序号 index | 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | U191D87 | D91D87 | Message of frame $1B9 such as HCU working status from HCU are lost | 来自HCU的制动踏板输入状态、油门踏板位置报文$1B9 丢失 | 开始监测：收到1F1报文钥匙位置为ON档后1000ms开始 停止监测：收到的1F1报文钥匙位置既非ON也非START或识别到1F1报文丢失100ms | 持续主动检测 continuous active testing | 连续250ms检测到该报文rolling counter错误（检测到报文丢失不应报此故障） ECU has detected message with rolling counter fault for consecutive 250 ms(detecting the loss of the message, it should not be reported) | 记录DTC | 连续5个报文周期未接收到该报文 rolling counter错误 ECU has detected message without rolling counter fault for 5 consecutive message periode | 故障灯不点亮 | 故障灯不点亮 | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

2.5.9.2DTCList(non network)

1.50A253

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A253 | 50A253 | DTC_ALIGNMENT_NOT_OK | 雷达下线标定未完成 | Drivetest is not active | 持续主动检测 | AlignmentState == ALIGNMENT_NOT_OK ¦¦ AlignmentState == ALIGNMENT_ONGOING | Record the DTC; Related function degrated | Alignment is done |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

2. 50A254

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A254 | 50A254 | DTC_ALIGNMENT_NEVER_DONE | 雷达从未下线标定 | Drivetest is not active | 持续主动检测 | AlignmentState == ALIGNMENT_NEVER_DONE | Record the DTC; Related function degrated | Alignment had been done |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

3. 50A255

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A255 | 50A255 | DTC_RADAR_NOT_MATCH | Radar实际安装位置与NVM里的标定位置不符 | Drivetest is not active | 持续主动检测 | After EOL and SDA is completed, write the position in to the NVM,and If radar position is different from the position in the NVM | Record the DTC; Related function degrated | radar position is match with the position in the NVM |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

4. 50A250

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A250 | 50A250 | DTC_RADAR_MOUNTING_ERROR | 雷达安装错误 | Drivetest is not active, Ecu start up for 3s | 持续主动检测 | when the radar is mounted, software will compare BODA information and PIN detection information, if two results are not consistent, error will be reported. | Record the DTC; Related function degrated | when the radar is mounted, BODA information and PIN detection information are match |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

5. 50A257

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A257 | 50A257 | DTC_ANTENNA_AUTHENTIC_BOOT_FAIL | 安全启动:雷达天线图完整性/真实性检查失效 | Drivetest is not active | 持续主动检测 | Antenna Diagram Trusted Boot check failed | Record the DTC; Related function degrated | Antenna Diagram Trusted Boot check passed |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

6. 50A278

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A278 | 50A278 | DTC_MRR_MISALIGNED | radar校准偏差过大 | Alignement progress completed | 持续主动检测 | radar misalignment horizontal angle is great than 5.7 degree radar misalignment vertical angle is great than 6.3 degree | Record the DTC; Related function degrated | Horizontal or vertical misalignment doesn't happen |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

7. 50A400

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A400 | 50A400 | DTC_CMRR_BLIND | radar系统失明 | 1> 3000ms after IGN ON 2> Production mode is swichted off 3> Drivetest is not active 4> No modulation off | 持续主动检测 | radar detect blindness | Record the DTC; Related function degrated | Sensor Blindness is not detected |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

8. 50A700

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A700 | 50A700 | DTC_PRODUCTION_MODE_ACTIVE | radar产线模式未关闭 | Drivetest is not active | 持续主动检测 | Production mode active | Record the DTC; Related function degrated | Production mode not active |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

9. 50A300

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A300 | 50A300 | DTC_FACTORYDATA_FAILURE | 雷达工厂数据错误 1)MMIC（单片微波集成电路)工厂数据不存在或数据版本号错误 (2) 雷达天线图不存在或版本号错误 以上任意出现2次 | 自检结束后开始 | 持续主动检测 | Factory data test Failure 工厂数据错误 | Record the DTC; Related function degrated | Replace hardware 更换硬件 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

10. 50A302

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A302 | 50A302 | DTC_INTERNAL_POWER_MANAGEMENT_SYS_FAILURE | 内部电源管理系统故障 | 自检结束后开始 | 持续主动检测 | ASIC Watchdog failure, The 10ms task of the operating system did not give feedback to watchdog normally ASIC Watchdog故障，操作系统10ms task未正常给Watchdog反馈 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

11. 50A307

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A307 | 50A307 | DTC_POWER_MANAGEMENT_CHIP_VOLTAGE_HIGH | 电源管理芯片过压故障 | 自检结束后开始 | 持续主动检测 | Power management chip over voltage fault(3.465V) Normal operating voltage 3.3V 电源管理芯片过压故障(3.465V) 正常工作电压3.3v | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

12. 50A306

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A306 | 50A306 | DTC_POWER_MANAGEMENT_CHIP_VOLTAGE_LOW | 电源管理芯片欠压故障 | 自检结束后开始 | 持续主动检测 | Power management chip under voltage fault (3.135V) Normal operating voltage 3.3V 电源管理芯片欠压故障(3.135V) 正常工作电压3.3v | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

13. 50A309

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A309 | 50A309 | DTC_SW_FAILURE | 软件故障 | 自检结束后开始 | 持续主动检测 | (1) Software task run time exceeds threshold（10ms/20ms） (2)Security management unit monitoring (SMU) is turned off (1) 软件任务运行时间超过阈值（10ms/20ms） (2) 安全管理单元监控（SMU）被关闭 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

14. 50A30C

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A30C | 50A30C | DTC_UC_TEMPERATURE_OUTOFRANGE | 微处理器温度超出范围故障 | 自检结束后开始 | 持续主动检测 | (1) The temperature of the microcontroller is beyond the reasonable measurement range of the sensor (below - 44 ° C or higher than + 145.9 ° C - > fail) (2) The main board temperature is out of the reasonable measurement range of the sensor (ADC group 0, channel 7 over + 128 ° C or below - 41 ° C → fail) (1) 微控制器温度超出传感器的合理测量范围（低于 -44°C或高于 +145.9°C -> fail) (2) 主板温度超出传感器的合理测量范围（ADC group 0, channel 7超过+128°C或低于 -41°C -> fail) | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

15. 50A30D

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A30D | 50A30D | DTC_BOSCH_DRIVETESTACTIVE | 内部驱动测试模式激活故障 | 自检结束后开始 | 持续主动检测 | Drive Test mode test failure 内部驱动测试模式激活故障 | Record the DTC; Related function degrated | Replace hardware 更换硬件 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

16. 50A30E

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A30E | 50A30E | DTC_INTERNAL_HW_FAILURE | 内部硬件故障 | 自检结束后开始 | 持续主动检测 | (monolithic microwave integrated circuit) internal hardware failure MMIC（单片微波集成电路) 内部硬件故障 | Record the DTC; Related function degrated | Replace hardware 更换硬件 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

17. 50A308

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A308 | 50A308 | DTC_RADAR_MODULATION_CONFIGURE | 雷达调制配置故障 | 自检结束后开始 | 持续主动检测 | Radar modulation configuration failure (MMIC register configuration) 雷达调制配置故障（MMIC寄存器配置） | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

18. 50A30F

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A30F | 50A30F | DTC_MMIC_TEMPERATURE_OVER_MAXIMUM | MMIC温度超过最大值故障 | 自检结束后开始 | 持续主动检测 | MMIC temperature exceeds maximum overheat error (130 ℃) MMIC温度超过最大值过热错误(130℃) | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

19. 50A401

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A401 | 50A401 | DTC_MMIC_TEMPERATURE_OVER_OPERATION | MMIC温度超过运行值故障 | 自检结束后开始 | 持续主动检测 | MMIC temperature exceeds operating value overheat error(125℃) MMIC温度超过运行值过热错误(125℃) | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

20. 50A304

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A304 | 50A304 | DTC_MMIC_VOLTAGE_HIGH | MMIC（单片微波集成电路)过压故障 | 自检结束后开始 | 持续主动检测 | Over voltage fault (MMIC) (3.465V) Normal operating voltage 3.3V MMIC（单片微波集成电路)过压故障(3.465V) 正常工作电压3.3v | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

21. 50A402

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A402 | 50A402 | DTC_RADAR_MODULATION | 雷达调制配置故障 | 自检结束后开始 | 持续主动检测 | The amplitude deviation of two radar channels exceeds the threshold 雷达两通道幅值偏差超过阈值 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

22. 50A305

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A305 | 50A305 | DTC_MMIC_VOLTAGE_LOW | MMIC欠压故障 | 自检结束后开始 | 持续主动检测 | Power management chip under voltage fault (3.135V) Normal operating voltage 3.3V MMIC欠压故障(3.135V) 正常工作电压3.3v | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

23. 50A30A

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A30A | 50A30A | DTC_SW_RIF_FAILURE | 雷达前端数据处理故障 | 自检结束后开始 | 持续主动检测 | Data consistency error of signal processing module (SPU) 信号处理模块（SPU）数据一致性错误 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

24. 50A303

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A303 | 50A303 | DTC_INTERNAL_UC_SYS_FAILURE | 内部微处理器系统故障 | 自检结束后开始 | 持续主动检测 | 1. CPU Loss of external clock signal 2. CPU Internal / external bus communication failure (1)CPU外部时钟信号丢失 (2)CPU核内/外设总线通信故障 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

25. 50A30B

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A30B | 50A30B | DTC_SW_TEMP_FAILURE | 软件临时故障 | 自检结束后开始 | 持续主动检测 | Unauthorized operations (such as tampering with special registers) have been detected for the current software running mode 检测到针对当前软件运行模式下存在未经授权的操作(如对特殊寄存器的篡改） | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

26. 50A200

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A200 | 50A200 | DTC_ADC_SELF_TEST_FAILURE | ADC自检错误 | 自检结束后开始 | 持续主动检测 | Radar ADC module self test failed 雷达ADC模块自检失败 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

27. 50A301

| 故障码 DTC | 故障码 （hex） | 故障码英文描述 English description | 故障码中文描述 Chinese description | 故障码运行条件 DTC operating conditions | 故障检测周期 DTC testing cycle | 故障码设置条件 DTC setting conditions | 故障码设置时发生的操作 Strategy after DTC appears | 故障码恢复条件 DTC resume conditions | 激活故障灯规则 Activate fault indicating lamp regulations | 故障灯熄灭规则 Fault indicating lamp Off regulations | 清除故障码条件 Clear fault information conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C10A301 | 50A301 | DTC_HW_UC_PFLASH | 微处理器程序校验故障 | 自检结束后开始 | 持续主动检测 | Application data integrity check failed 应用程序数据完整性校验失败 | Record the DTC; Related function degrated | 1. Perform radar power reset by unplugging ECU itself or corresponding power fuse 2. Power on the vehicle again and wait for 5 seconds 3. Clear the DTC 4. Read DTC If there is no DTC: Radar can be reused If the DTC is still present / active: 5. Replace HW 1.通过拔下ECU本身或相应的电源保险丝来执行雷达电源重置 2.车辆重新上电，等待5秒钟 3.清除DTC 4.读取DTC 如果不存在DTC： 雷达可复用 如果DTC仍然存在/处于活动状态： 5.更换雷达 |  |  | 1、经过40次无故障操作循环(点火钥匙从ON至OFF)后，历史故障诊断码将被清除。 2、控制模块接收到一个由诊断工具发出的清除故障码的指令。 1. After 40 non-fault operating cycles(from ignition on to ignition off), the historical DTCs shall be clearified. 2. ECU receives a command of clearing DTCs from the tester. |

2.5.10NRC：

7F/12/13/31

2.5.11NRC优先级（待澄清）

2.6 0x22

2.6.1诊断属性要求：

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 22 | ReadDataByIdentifier 读取数据（通过本地标识） |  | Y | Y | Y | Y | N | N |

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 22 | ReadDataByIdentifier 读取数据（通过本地标识） |  | Y | Y | Y | Y | N |

2.6.3支持的DID数量

5个

2.6.4未使用位填充值

所有ASCII编码的通用DID，未使用的字节只能用字符0（即0x30）填充。

2.6.5NRC：

13/31

2.6.6NRC优先级（待澄清）

2.7 0x2E

2.7.1诊断属性要求：

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 2E | WriteDataByIdentifier 写入数据（通过标识） |  | N | N | Y | N | Level1 | N |

2.7.2时间及存储位置要求：

通过本服务写入的DID数值应立即保存结果至EEPROM，不允许出现下电后才开始进行保存操作的情况。

2.7.3次数要求：

在EEPROM寿命允许下，所有支持写的DID，应支持不限制次数的写操作。

2.7.4肯定响应要求：

当执行本服务涉及写EEPROM操作时，必须要在完成全部EEPROM操作后，才能给出最终响应，若无法在规定时间内给出最终响应，应该启用增强型定时参数（NRC78）。

2.7.5NRC：

7F/13/31/33

2.7.6NRC优先级（待澄清）

2.8 SID22&2E支持的DID

F180:

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F180 | BootSoftwareIdentification | 供应商boot软件标识符 | 13 | ASCII | Y | N | 实际值 |

F184

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F184 | applicationSoftwareFingerprint | 应用程序指纹记录 | 9 | USN | Y | level1 |  |

F186

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F186 | Active Diagnostic Session | 激活的诊断会话 | 1 | USN | Y | N | 默认值：01 01：默认会话 02：编程会话 03：扩展会话 |

F188

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F188 | SGMW ECU Software Number | 五菱ECU软件号 | 4 | USN | Y | N |  |

F18A

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F18A | Supplier Identifier Information | 供应商标识信息 | 7 | ASCII | Y | N | 实际值 |

F18B

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F18B | ECUManufacturingDate | ECU生产日期 | 4 | BCD | Y | N | 实际值 |

F18C

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F18C | ECU Serial Number | ECU序列号 | 40 | ASCII | Y | N | 实际值 |

F190

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F190 | SGMW VIN | SGMW车辆标识码 | 17 | ASCII | Y | level1 | 全0x00 |

F191

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F191 | SGMW ECU Hardware Number | 五菱ECU硬件号 | 4 | USN | Y | N |  |

F192

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F192 | Supplier ECU Hardware Number | 供应商ECU硬件号 | 5 | ASC¦¦ | Y | N |  |

F193

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F193 | Supplier ECU Hardware Version Number | 供应商ECU硬件版本号 | 13 | ASCII | Y | N | 实际值 |

F194

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F194 | Supplier ECU Software Number | 供应商ECU软件号 | 4 | ASC¦¦ | Y | N |  |

F195

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F195 | Supplier ECU Software Version Number | 供应商ECU软件版本号 | 13 | ASCII | Y | N | 实际值 |

F1CB

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F1CB | EndModelPartNumber(SGMW P/N) | ECU产品级零件号 | 4 | USN | Y | N |  |

F1C1

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F1C1 | SGMW ECU ApplicationSoftware Version Number | 五菱ECU应用软件版本号 | 18 | ASCII | Y | N |  |

2600

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2600 | Production Mode | 产线模式 | 1 | 0 | 1 | —— | 0x00: OFF（ACC/AEB/FCW功能开启） 0x01: ON（ACC/AEB/FCW功能抑制） | HEX | Y | level1 | 0x01: ON | If this value is 0x01, a DTC C212F06 should be set. After this value is changed to 0x00, the DTC should become a historical DTC in current operating cycle |

2601

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2601 | SCU Temperature ECU | 温度 | 1 |  |  |  | Temperature = XX * 1 - 50 ℃ | Unsigned | Y | N | 0x00 |  |

2606

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2606 | Horizontal Slow Misalignment Angle | 水平慢速偏差角度 | 4 |  |  |  | Degree = xx * 57.2958 / 33554432 + 0 Deg, | Signed | Y | N | 0x00 |  |

2607

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2607 | Horizontal Fast Misalignment Angle | 水平快速偏差角度 | 4 |  |  |  | Degree = xx * 57.2958 / 33554432 + 0 Deg, | Signed | Y | N | 0x00 |  |

2608

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2608 | DeviationAngleHorizontal_EOL | 主动式校准水平偏差角度 | 2 |  |  |  | Angle = 57.2958 / 16384 * xx + 0 | Signed | Y | N | 0x00 |  |

2609

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2609 | DeviationAngleVertical_EOL | 主动式校准垂直偏差角度 | 2 |  |  |  | Angle = 57.2958 / 16384 * xx + 0 | Signed | Y | N | 0x00 |  |

2611

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2611 | Vertical Slow Misalignment Angle | 垂直慢速偏差角度 | 4 |  |  |  | Degree = xx * 57.2958 / 33554432 + 0 Deg, | Signed | Y | N | 0x00 |  |

2612

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2612 | Vertical Fast Misalignment Angle | 垂直快速偏差角度 | 4 |  |  |  | Degree = xx * 57.2958 / 33554432 + 0 Deg, | Signed | Y | N | 0x00 |  |

2802

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2802 | InvalidKeyCounter | 无效密钥计数 | 1 |  |  |  |  | Unsigned | Y | N | 0x00 |  |

2805

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| 2805 | VehicleDataInformation | 车辆信息 | 41 |  |  |  | Byte Name 0 dVehicleWidth_HighByte (最大车宽) 1 dVehicleWidth_LowByte 2 sensorOrientation (雷达安装方向) 3 lWheelbase_HighByte (轴距) 4 lWheelbase_LowByte 5 lWheelTrack_HighByte (轮距) 6 lWheelTrack_LowByte 7 vChar_HighByte (特征车速) 8 vChar_LowByte 9 dVehicleWidthMin_HighByte (最小车宽) 10 dVehicleWidthMin_LowByte 11 dCustomer2MrrXAxisOffset_HighByte (车身坐标系和雷达坐标系在X方向的距离偏差 ) 12 dCustomer2MrrXAxisOffset_LowByte 13 dCustomer2MrrYAxisOffset_HighByte (车身坐标系和雷达坐标系在Y方向的距离偏差 ) 14 dCustomer2MrrYAxisOffset_LowByte 15 dCustomer2MrrZAxisOffset_HighByte (车身坐标系和雷达坐标系在Z方向的距离偏差 ) 16 dCustomer2MrrZAxisOffset_LowByte 17 dCustomerMountPosX_HighByte (雷达和车身坐标系原点在X方向的距离) 18 dCustomerMountPosX_LowByte 19 dCustomerMountPosY_HighByte (雷达和车身坐标系原点在Y方向的距离) 20 dCustomerMountPosY_LowByte 21 dCustomerMountPosZ_HighByte (雷达和车身坐标系原点在Z方向的距离) 22 dCustomerMountPosZ_LowByte 23 WheelAngleRatioCharacteristics_HighByte (转向比) 24 WheelAngleRatioCharacteristics_LowByte 25 dVehicleMass_HighByte (车辆质量) 26 dVehicleMass_LowByte 27 dWheelRadius_HighByte (车轮滚动半径) 28 dWheelRadius_LowByte 29 dDxvRearBumper_HighByte (后悬) 30 ddxvRearBumper_LowByte 31 dxvFrontBumper_HighByte (前悬) 32 dxvFrontBumper_LowByte 33 dCenterPillar_HighByte 34 dCenterPillar_LowByte 35 dMountAngleAzimuth_HighByte 36 dMountAngleAzimuth_LowByte 37 dMountAngleElevation_HighByte 38 dMountAngleElevation_LowByte 39 dxvMountPosAxSensor_HighByte 1)Formula: phys = (1 / 128) * (signed_16Bits)Data + 0 2)Formula: phys = 1 * (signed_8Bits)Data + 0 3)Formula: phys = (1 / 2048) * (signed_16Bits)Data + 0 4)Formula: phys = (1 / 256) * (signed_16Bits)Data + 0 5)Formula: phys = (1 / 2048) * (unsigned_16Bits)Data + 0 6)Formula: phys = 1 * (signed_16Bits)Data + 0 | HEX | Y | N | Based on the defination of VKI file. |  |

CF00:

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| CF00 | Read ECU Lifecycle | 读取ECU生命周期 |  |  |  |  | Return value 返回值: 0x0a: Shipping 运输 0x0b: Series 量产 0x0c: Others state 其他状态 0x0d: Too frequent access need to wait 30s 请求太频繁，等待30s. 0x13, 0x14, 0x31, , 0x33, 0xF1 - 0xFA : other negative return value 其他负反馈值 Note: 22 service for read, 2E service for write. 22读取服务； 2E写入服务。 | ENUM | Y | N | 0x00 |  |

CF01

| DID number DID号 | DID English Description DID英文描述 | DID Chinese Description DID 中文描述 | Size(Bytes) 字节数 | Range, Min (Phy.) | Range, Max (Phy.) | Unit 单位 | Conversion 换算 | Data Type 数据类型 | Service $22 Access $22服务访问 | Service $2E Access $2E服务访问 | Default 默认值 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 物理值最小值 | 物理值最大值 |  |  |  |  |  |  |  |
| CF01 | ECU Lifecycle Switch(series -> shipping) | ECU生命周期切换（量产->运输） | 1 |  |  |  | Parameter: 0x00: lifecycle switch off/生命周期关闭 0x01: lifecycle switch from series to shipping/生命周期切换从量产到发货 Return Value 返回值 : 0x00: E_OK (no errors occurs) 正反馈 无错误 0x0d: Too frequent access need to wait 30s 请求太频繁，等待30s. 0x13: incorrectMessageLengthOrInvalidFormat 信息长度错误 或 无效格式 0x22: INFIELD status not INFIELD2 内场状态 非内场2 0x31: requestOutOfRange 请求超过范围 0x33: securityAccessDenied 安全权限禁止 0x72: generalProgrammingFailure 刷新错误 0xF1 - 0xFA : other negative return value 其他负反馈值 | ENUM | Y | Y | 0x00 |  |

2.9 0x27

2.9.1安全等级

在一种会话模式下，任何时刻仅应有一个安全等级处于激活状态。

2.9.2诊断属性要求

Level 1:

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 27 | SecurityAccess 安全访问 | 01/02 general security-level1 一般安全 | N | N | Y | N | N | N |

Level 3:

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 27 | SecurityAccess 安全访问 | 05/06 programming security-level3 编程安全 | N | N | Y | N | N |

2.9.3安全算法存储

安全算法应存储在非易失性存储器内。

2.9.4 失败计数器 failure counter

在解锁过程中，失败计数器用于记录解锁失败次数

2.9.4.1失败计数器最大值

失败计数器最大值为3.

2.9.4.2失败计数器存储

失败计数器应该存储在ECU的非易失性存储器内，该非易失性存储器不应由于ECU上电/断电以及任何类型的复位而被初始化。

2.9.4.3失败计数器读取

每当上电/复位后，ECU均应检测失败计数器的数值。

2.9.4.4失败计数器增加

当收到一个无效Key，失败计数器加一。

2.9.4.5失败计数器清零

在3分钟的延时完成或者一次成功的“发送密钥”安全访问执行后，ECU内部的失败计数器应清零。

2.9.4.6切换会话模式对失败计数器的影响

切换会话模式，原会话模式下对应的安全访问等级下的失败计数器数值不变。

2.9.4.7出厂设置

ECU在供应商出厂时，失败计数器的值应置零。

2.9.4.8不同安全解锁等级之间对失败计数器的影响

每个会话模式下的每个安全访问等级的失败计数器应分别处理

2.9.5延时时间

当失败计数器达到一定的值，需要激活延时时间。

2.9.5.1延时时间

3分钟/180秒

2.9.5.2激活延时时间条件

当失败计数器达到3时，激活延时时间。

2.9.5.3切换会话对延时时间的影响

切换当前的会话模式，原会话模式下的安全等级下的延时时间不受影响。

2.9.5.4上电/复位对延时时间的影响

每当上电/复位后，ECU均应检测失败计数器的数值，若其值为3，则ECU应立即激活3分钟的延时。

2.9.5.5不同安全解锁等级之间对延时时间的影响

每个会话模式下的每个安全访问等级的3分钟延时应分别处理。

2.9.6解锁状态

2.9.6.1解锁成功

当接收到正确的密匙，ECU解锁成功。

2.9.6.2 切换会话模式，解锁状态失效

在解锁期间，切换当前的会话模式（包括当前激活的会话），ECU重新进入到锁定状态。

2.9.6.3 上电/复位，解锁状态失效

在解锁期间，成功上电/复位之后，ECU应锁定所有会话模式下的所有安全等级。

2.9.6.4 当S3Server超时，解锁状态失效

在解锁期间，未收到任何诊断请求去重启S3Server，那么S3Server超时，ECU重新进入到锁定状态。

2.9.7安全种子

2.9.7.1种子生成

安全种子是ECU随机生成的4个byte的随机数。

2.9.7.2 种子内容的要求

在ECU的肯定响应消息中种子的值不允许为FFFFFFFFF（hex）。

2.9.7.3当ECU锁定状态去请求种子

1.请求一次种子：

每次请求种子时ECU软件内部就会产生一个随机的种子,但是值不应该是0x00000000.

2.多次尝试请求种子（待澄清）

2.9.7.4当ECU解锁状态去请求种子

当接收到同安全等级的一个“请求种子”时，那么ECU应发送一个种子值为“0x00000000”的肯定响应消息。

2.9.8安全密钥

2.9.8.1密钥生成

通过完全算法，用获取的种子计算出唯一的密匙，其长度为4个byte。

2.9.9 有效密钥情况下的安全访问流程

2.9.10 无效密钥情况下的安全访问流程

2.9.11NRC：

7F/12/13/24/35/36/37

2.9.12 NRC优先级（待澄清）

2.10 0x28

2.8.1通讯类型：（待澄清）

2.8.2 28 00  使能收发

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 28 | CommunicationControl 通信控制 | 00 enableRxAndTx/使能收发 | N | N | Y | Y | N | N |

2.8.3 28 03禁止收发

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 28 | CommunicationControl 通信控制 | 03 disableRxAndTx/禁止收发 | N | N | Y | Y | N | N |

2.8.4 切换会话模式，对通讯控制的影响

在新的会话模式下仍支持，则被激活的通讯控制状态应继续维持；

在新的会话模式不支持，则被激活的通讯控制状态应失效。回到ECU默认状态，即 TX RX 均恢复通信 。

2.8.5 NRC：

7F/13/12/31/22

NRC22:车速大于10km/h不允许正确执行，返回NRC22否定码

2.8.6 NRC优先级（待澄清）

2.11 0x31

2.11.1 31 01开始程序

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 31 | RoutineControl 例程控制 | 01 startRoutine开始程序 | N | N | Y | N | N | N |

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 31 | RoutineControl 例程控制 | 01 startRoutine开始程序 | N | N | Y | N | Level3 |

2.11.2 31 02停止程序

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 31 | RoutineControl 例程控制 | 02 stopRoutine停止程序 | N | N | Y | N | N | N |

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 31 | RoutineControl 例程控制 | 02 stopRoutine停止程序 | N | N | Y | N | Level3 |

2.11.3 31 03请求程序结果

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 31 | RoutineControl 例程控制 | 03 requestRoutineResults请求程序结果 | N | N | Y | N | N | N |

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 31 | RoutineControl 例程控制 | 03 requestRoutineResults请求程序结果 | N | N | Y | N | Level3 |

2.11.4 切换会话模式，对例程控制的影响

在新的会话模式下仍支持，则被激活的例程控制状态应继续维持；

在新的会话模式下不支持，则被激活的例程控制状态应失效。

2.11.5 支持的RID

1.售后校准0x0308

| Routine Identifier(HEX) 例程RID | RID English Description RID英文描述 | RID Chinese Description RID中文描述 | RoutineControlType(HEX) 例程控制类型 | RoutineControlOptionRecord(HEX) 例程控制选项记录 | RoutineStatusRecord(HEX) 例程状态记录 | Security Level(HEX) 安全等级 | Suport Diagnostic Session(HEX) 支持的诊断会话 |  | Operation Conditions 例程执行条件 | Examples / 指令举例 | Remarks 备注 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 02 ProgrammingSession/ 编程会话模式 | 03 ExtendedDiagnosticSession/ 扩展诊断会话模式 |  |  |  |  |
| ECU Supplier Specific RIDs (0x0200 - 0xDFFF, 0xF000 – 0xFEFF) - Service $31 |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x0308 | Service Drive Alignment | 售后校准 （可忽略“车速大于10km/h不允许正确执行”的原则） | 01startRoutine | N/A | N/A | N | N | Y | 备注：若雷达安装错误，ECU则会回复NRC94 | Request : 31 01 03 08 Response : 71 01 03 08 XX XX definition 0x00=calibration is successfully executed 0x01=calibration is processing 0x02=calibration failed 0x00= 校准已成功执行 0x01= 校准正在进行中 0x02= 校准失败 | "Service Drive Alignment Routine For 03: RequestRoutineResults, there are 8 bytes response from Radar, the meaning for each byte as follow: X1 : Calibration Result 0x00=successfully calibrated 0x01=calibration is processing 0x02=calibration is failed 0x03= cancelled by user 0x04~0xFE=reserved 0xFF=routine not started X2 : Calibration Schedule : progess bar 0~100% (Resolution=1, Offset=0) X3: Cause of Calibration failure: 0x00=Calibration succeeded 0x01=Calibration time out 0x02=Vertical/Horizontal angle out of range 0x03=NVM write error 0x04= Cancelled by user 0x05= SDA calibration failed 0x06~0xFE=Reserved 0xFF=Calibration not started X4 X5: Horizontal deviation angle : range (--3,3) degree . (Resolution=0.01, Offset=0) type : Hex (signed) X6 X7 : Vertical deviation angle: 0x0000 . Corner HW not support vertical alignment X8 : Driving Profile : Bit 0 : 0x00：Calibration requirements ok 0x01：Driving too slow(< 10km/h) Bit 1: 0x00：Calibration requirements ok 0x01：Driving too fast(>250km/h) Bit 2: 0x00：Calibration requirements ok 0x01：Yaw rate too big(>0.1 rad/s or <-0.1 rad/s) Bit 3 :0x00：Calibration requirements ok 0x01：Longitudinal acceleration too large(>5 m/s^2 or <-5 m/s^2 ) Bit 4: 0x00：Calibration requirements ok 0x01： location number not sufficient Bit 5 : 0x00：Calibration requirements ok 0x01：sensor is blind Bit6~Bit7: reserved | 对于03 ： RequestRoutineResults ， 雷达有 8 个字节的响应， 每个字节的含义如下： X1 ：校准结果 0x00= 校准成功 0x01= 校准正在进行中 0x02= 校准失败 0x03= 被用户取消 0x04 取消~0xFE= 预留 0xFF= 例程未启动 X2 ：校准进度： 程序条 0~100% (分辨率 =1 ，偏移 =0) X3 ： 校准失败的原因： 0x00= 校准成功 0x01= 校准超时 0x02= 垂直 / 水平角度超出范围 0x03= NVM 写入错误 0x04= 被用户取消 0x05= SDA 校准失败 0x06~0xFE= 预留 0xFF= 校准未开始 x4 X5 ：水平偏差角度： 范围 (-3,3) 度。 (分辨率 =0.01 ，偏移量 =0) X6 X7 ：垂直偏差角度： 0x0000。 角硬件不支持垂直对齐 x8 ：驾驶模式： Bit 0 ： 0x00：校准要求正常 0x01：行驶速度太慢 (< 10 公里 / 小时) Bit 1 ： 0x00：校准要求正常 0x01：行驶速度过快 (>250 公里 / 小时) Bit 2 ： 0x00：校准要求正常 0x01：偏航率太大 (>0.1 弧度 / 秒或 <-0.1 弧度 / 秒) Bit 3 ： 0x00：校准要求正常 0x01：纵向加速度过大 (>5 m/s^2 或 <-5 m/s^2) Bit 4 ： 0x00：校准要求正常 0x01： 位置编号不足 Bit 5 ： 0x00：校准要求正常 0x01：传感器为遮阳帘 Bit6~Bit7 ：预留 |
|  |  |  | 02stopRoutine | N/A | N/A |  |  |  |  | Request : 31 02 03 08 Response : 71 02 03 08 |  |  |
|  |  |  | 03requestRoutineResults | N/A | "Service Drive Alignment Routine For 03: RequestRoutineResults, there are 8 bytes response from Radar, the meaning for each byte as follow: X1 : Calibration Result 0x00=successfully calibrated 0x01=calibration is processing 0x02=calibration is failed 0x03= cancelled by user 0x04~0xFE=reserved 0xFF=routine not started X2 : Calibration Schedule : progess bar 0~100% (Resolution=1, Offset=0) X3: Cause of Calibration failure: 0x00=Calibration succeeded 0x01=Calibration time out 0x02=Vertical/Horizontal angle out of range 0x03=NVM write error 0x04= Cancelled by user 0x05= SDA calibration failed 0x06~0xFE=Reserved 0xFF=Calibration not started X4 X5: Horizontal deviation angle : range (--3,3) degree . (Resolution=0.01, Offset=0) type : Hex (signed) X6 X7 : Vertical deviation angle: 0x0000 . Corner HW not support vertical alignment X8 : Driving Profile : Bit 0 : 0x00：Calibration requirements ok 0x01：Driving too slow(< 10km/h) Bit 1: 0x00：Calibration requirements ok 0x01：Driving too fast(>250km/h) Bit 2: 0x00：Calibration requirements ok 0x01：Yaw rate too big(>0.1 rad/s or <-0.1 rad/s) Bit 3 :0x00：Calibration requirements ok 0x01：Longitudinal acceleration too large(>5 m/s^2 or <-5 m/s^2 ) Bit 4: 0x00：Calibration requirements ok 0x01： location number not sufficient Bit 5 : 0x00：Calibration requirements ok 0x01：sensor is blind Bit6~Bit7: reserved |  |  |  |  | Request : 31 03 03 08 Response : 71 03 03 08 X1 X2 X3 X4 X5 X6 X7 X8 |  |  |

2.下线标定 0x4F68

| Routine Identifier(HEX) 例程RID | RID English Description RID英文描述 | RID Chinese Description RID中文描述 | RoutineControlType(HEX) 例程控制类型 | RoutineControlOptionRecord(HEX) 例程控制选项记录 | RoutineStatusRecord(HEX) 例程状态记录 | Security Level(HEX) 安全等级 | Suport Diagnostic Session(HEX) 支持的诊断会话 |  | Operation Conditions 例程执行条件 | Examples / 指令举例 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 02 ProgrammingSession/ 编程会话模式 | 03 ExtendedDiagnosticSession/ 扩展诊断会话模式 |  |  |  |
| ECU Supplier Specific RIDs (0x0200 - 0xDFFF, 0xF000 – 0xFEFF) - Service $31 |  |  |  |  |  |  |  |  |  |  |  |
| 0x4F68 | EOL Alignment |  | 01startRoutine | N/A | N/A | N | N | Y | 备注：若雷达安装错误，ECU则会回复NRC94 如果车速大于10km/h，则会报出NRC22，NRC94优先级高于NRC22 | Request : 31 01 4F 68 Response : 71 01 4F 68 XX XX definition as below 0x00=calibration is successfully executed 0x01=calibration is processing 0x02=calibration failed |  |
|  |  |  | 02stopRoutine | N/A | N/A |  |  |  |  | Request : 31 02 4F 68 Respnse : 71 02 4F 68 | Because the timming to perform EOL just 10s. So suggest remove service 02 to stop alignment 因为执行 EOL 的时间仅为 10 秒。 因此建议删除 service02 |
|  |  |  | 03requestRoutineResults | N/A | The meaing for each byte as follows: X1: Bit0-Bit3, Routine Status; 0x00: Routine inactive; 0x01: Routine active; 0x02: Routine NVM write not OK; 0x03: Routine timeout; 0x04: Routine finished correctly; 0x05: Routine aborted; Bit4-Bit7: Routine Result; 0x0: Alignment no result available; 0x1: Alignment incorrect result; 0x2: Alignment correct result; X2 X3: Horizontal deviation angle, Factor: 0.01 ; type : Hex (signed) X4 X5: Vertical deviation angle, Factor: 0.01 . 每个字节的含义如下： X1 ： Bit0-Bit3 ，例程状态； 0x00 ：例行程序未激活； 0x01 ：例程处于激活状态； 0x02 ：例程 NVM 写入不正确； 0x03 ：例程超时； 0x04 ：例行程序已正确完成； 0x05 ：例程已中止； Bit4-Bit7 ：例程结果； 0x0 ：无校准结果； 0x1 ：校准不正确； 0x2 ：校准正确； X2 X3 ：水平偏差角度，因子： 0.01 ； x4 X5 ：垂直偏差角度，系数： 0.01。 |  |  |  |  | Req ：31 01 4F 68 Resp ：71 01 4F 68 Req ： 31 03 4F 68 Resp： 71 03 4F 68 X1 X2 X3 X4 X5 The meaning for each byte as follows: X1: 0x00=calibration successful 0x01=calibration in progress 0x02=calibration timeout 0x03=vertical angle deviation too large 0x04=horizontal angle deviation too large 0x05=target lost 0x06=Write NVM error X2 X3: Horizontal deviation angle : range (--3,3) degree . (Resolution=0.01, Offset=0) type : Hex (signed) X4 X5 : Vertical deviation angle: 0x0000: Corner HW not support vertical deviation angle | 0x00= 校准成功 0x01= 校准进行中 0x02= 校准超时 0x03= 垂直角度偏差太大 0x04= 水平角度偏差太大 0x05= 目标丢失 0x06= 写入 NVM 错误 范围 (-3,3) 度。 (分辨率 =0.01 ，偏移量 =0) x4 X5 ：垂直偏差角度： 0x0000 ：角硬件不支持垂直偏差角度 |

2.11.6NRC：

7F/13/31/33/12/24/22

NRC22:车速大于10km/h不允许正确执行，返回NRC22否定码

2.11.7 NRC优先级（待澄清）

2.12 0x3E

2.12.1 诊断属性要求：

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 3E | TesterPresent 诊断设备在线 | 00 (suppressPosRspMsgIndicationBit =0)/不抑制正响应 | Y | Y | Y | Y | N | N |
|  |  | 80 (suppressPosRspMsgIndicationBit =1)抑制正响应 | Y | Y | Y | Y | N | N |

2.12.2 NRC：

12/13

2.13.3 NRC优先级（待澄清）

2.13 0x85

2.13.1 85 01使能

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 85 | ControlDTCSetting 控制DTC设置 | 01 ON使能 | N | N | Y | Y | N | N |

2.13.2 85 02禁止

| Diagnostic Questionnaire 1-Diagnostic Services(Application software) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 （应用软件） |  |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  | Security Access 安全访问 |  |
|  |  |  | Default 默认 |  | Extended 扩展 |  | Level1 层级1 | Level2 层级2 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |  |
| 85 | ControlDTCSetting 控制DTC设置 | 02 OFF禁止 | N | N | Y | Y | N | N |

2.13.3 切换会话模式，对DTC控制状态的影响

在新的会话模式下仍支持，则被激活的DTC控制状态应继续维持；

在新的会话模式不支持，则被激活的DTC控制状态应失效。回到ECU默认状态，即 DTC正常报出。

2.14.4 NRC：

7F/13/12

2.14.5 NRC优先级（待澄清）

2.14 0x34

2.14.1 诊断属性要求：

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 34 | RequestDownload 请求下载 |  | N | N | Y | N | Level3 |

2.14.2 NRC：

13/31/33/70/78

2.14.3 NRC优先级（待澄清）

2.15 0x36

2.15.1 诊断属性要求：

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 36 | TransferData 发送数据 |  | N | N | Y | N | N |

2.15.2 NRC：

13/31/33/70/78

2.15.3 NRC优先级（待澄清）

2.16 0x37

2.16.1 诊断属性要求：

| Diagnostic Questionnaire 1-Diagnostic Services(boot software) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 诊断调查问卷1 - 诊断服务 (引导加载程序) |  |  |  |  |  |  |  |
| Service ID 服务标识符(HEX) | Diagnostic Services Name 诊断服务名称 | Sub-functions 子功能(HEX) | Session/会话 |  |  |  |  |
|  |  |  | Default 默认 |  | Programming 编程 |  | Level3 层级3 |
|  |  |  | Phys 物理 寻址 | Func 功能 寻址 | Phys 物理 寻址 | Func 功能 寻址 |  |
| 37 | RequestTransferExit 请求退出发送 |  | N | N | Y | N | N |

2.16.2 NRC：

13/24

2.16.3 NRC优先级（待澄清）